“Linear Optimal Control Systems”, Kwakernaak, Sivan, 1972

- Elements of Linear System Theory

  - Introduction

  - A State description of Linear System

- Ex.1.1 (Kwak. Page 4): Inverted pendulum: stabilization of the inverted pendulum

To stabilize the pendulum to move the carriage

- Parameters of Modeling

- A small motor force to the carriage: u(t)

- The displacement of the pivot: s(t)

- Angular rotation of the pend: ϕ(t)

- The mass of Pend.: m

- The distance from the pivot to the center of gravity: L

- The moment of inertia: J

- The carriage mass: M

- The force on the Pend:mg

- The horizon reaction force: H(t)

- The vertical reaction force:V(t)

- The horizontal friction between the carriage and the road :F

- No friction at the pivot

%%%%%%%%%%%%%%%%----- application of an inverted pendulum - google

---------------------------%%%%%%%%%%%%%%%%%%%

- Dynamics at the center of the Pend.

md2dt2st+Lsinϕt =Ht     (1.15)

md2dt2Lcosϕt =Vt-mg                 (1.16)

Jd2ϕtdt2=LVsinϕt-LHtcosϕt    (1.17)

Md2stdt2=ut-Ht-Fdstdt                (1.18)

- Assumption:

- m≪M

Ms=ut-Ht-Fs          (1.19)

- elimination of V,H in (1.16),(1.17) and (1.18)

J+mL2ϕ-mgLsinϕ+mL scosϕ=0           (1.20)

which yields to 
ϕ-gL'sinϕ+1L' scosϕ=0                               (1.21)

where L'=(J+mL2)/mL : effective pendulum length

- For linearization, choose a nominal solution as u=0,s=0,ϕ0=0, hence

ϕ-gL'ϕ+1L's=0                 (1.23)

- Define new states for the linearized dynamic system equation (1.23).

ξ1t=st, ξ2t= st, ξ3t=st+L'ϕt, ξ4t= st+L'ϕ(t),

Then the dynamics of the system is

xt= 01000-FM000-gL'000gL'10xt+ 01M00ut            (1.26)

You may check ϕ=(ξ3-ξ1)/L'

The numerical values are

FM=1 s-1,1M=1 kg-1, gL'=11.65 s-2, L'=0.842 m

- Home assignment:

From (1.15),(1.16),(1.17) and (1.18), derive (1.26)

%%%%%%%%%%%%------During this semester we will study “Optimal Control” and introduce example this inverted Pendulum inverted pendulum

- modelling

- stability / controllability / observability

- steady state analysis

- optimal regulator design

- Stochastic modelling

- Optimal estimator / optimal controller / separation principle

- Optimal output feedback(in complete measurements)

-------------------------%%%%%%%%%%%%%%

  - Solution of the state differential equation of linear system

    - The transition Matrix and the impulse response matrix

    - The transition matrix of a Time-invariant system

- Theorem 1.4. (P.14)

- The solution of (1-67) is

xt=Φt,t0 xt0= eAt-t0x(t0)

- Using theorem 1.3(p.12), if the new state yt=Cxt, then

x=Ax+Bu, 
y=cx

Then
yt=Cetx0+C0teAt-τBuτdτ                               (1-70)

  - Stability

- Def.1.1 (p.24) : stability in the sense of Lyapunov

- Def 1.2. Asymptotically stability

- Theorem 1.14 (p.28): stability of The time-invariant linear system

- Def. 1.6 : characteristic values(=eigen-values)

- Ex.1.11 (p.34, the inverted pendulum)

Find the characteristic values. Is it asymptotically?

  - Transform analysis of time-invariant systems

    - Solution of the state differential equation through Laplace Transformation

- Fact(P.33):

- Laplace transform

Zs= Lzt= 0∞e-stztdt                                       (1-150)

- The solution of homogeneous LTI

x=Ax                                                       (1-151)

The Laplace transform is

Xs=sI-A-1x(0)

The inverse Laplace transform is

xt=eAtx0                                                     (1-154)

- The solution of inhomogeneous LTI

x=Ax +Bu, y=Cx                                        (1-163)

The Laplace Transform is

Ys=CXs=CSI-A-1x0+CsI-A-1BUs        (1-167)

- The transfer matrix(function) from u to y

Hs=YsUs ,     Hs=CsI-A-1B                           (1-170)

- Def: The poles

The roots of the common denominator of H(s) are called the poles of the transfer function.

  - Sfda

Sda

A linear time-varying system,

xt=Atxt+Btu(t)

to implement a linear control law

ut= -Ftxt+u'(t)

Ft: a time-varying state feedback gain matrix

u't: a new input

The closed loop system is asymptotically stable

xt=At-BtFtxt+Btu'(t)

%%%%%%%%%%-----

Asymptotically: a line approaches a curve but never touches.

xt=e-tx0,   limt→∞xt=0,   but ∀t , xt≠0, if x0≠0

-------------- %%%%%%%%%%%%%%%