- Review the last lecture

The main topic is “controllability”, which was introduced Kalman at first. As you see the linear system

x=Ax+Bu

Now in order to assign any pole as you like, mathematicians thinks the sufficient condition of the controllability is the input matrix "B" is full rank,

u=KBx= K1⋯0⋮⋱⋮0⋯1x1⋮xn
=k11⋮0x1+ k2010x2+,…+kn0⋮1xn

Yap. That’s a common sense. But the drawback is  You need the number of the controllers(i.e., actuators, in mechanics in general) is  “n” , which is TOO many controllers.

In the previous example 1.20, the inverted pendulum, the controller u(t) is

ut=k-ξ1+ξ3= k-1010Tξ1ξ1ξ3ξ4

which means only the one controller guarantee the controllability !, which is one of the main contribution of Kalman.

- Home assignment: W6.5, If B = 0 0 0 1', is the system is controllable?

  - Reconstructibility

In most control textbook, the observability is used instead of the reconstructability.

- Def. Observability (1-335)

The system (1-328) is said to be completely observable if for all t0 there exists a t1<∞ such that

yt;t0,x0, u=yt;t0,x0', u,   t0≤t≤t1                (1-335)

for all ut, t0≤t≤t1, implies that x0=x0'

%%%%%%%%%%%--------- what is meaning on the observability?

It is more common to deal with time-invariant systems in this course, i.e. A,B,C are constant w.r.t. time. Then the system is observable,

If the system is time-invariant, i.e. A,B,C are constant w.r.t time, for any t1

yt;t0,x0, u=yt;t0,x0', u,   t0≤t≤t1             (1-335.1)

What does it mean?  Let ut=0, and t0=0. Then if the system is observable, for any t

yt, x0=yt,x0'

which implies, measured output yt is uniquely determine x0. You should  remember x0∈Rn

x0= ξ1(0)⋮ξn(0)
.

Hence given yt you may calculate the exact value of xt1,  t0≤ t1≤t, not estimate of x(t).

- Theorem 1.32 (The  Reconstructibility (the observability) condition)

%%%%%%%%%--------------- example. Let the position ξt of a vehicle with a constant acceleration a is modelled as

ξ=a, -→ξt=ξ0+ ξ0t+12at2, a is known
The corresponding state space model may be defined as   ξt=x1t, ξ=x2

x1x2=0100x1x2+ 0a,

- Case _1 : If your measurement is the position, x1t, ∀ t,   i.e.,

yt=x1t= 1 0x1x2=Cxt.

Hence the reconstructibility(observability) matrix is

Q= CCA= CCA=1001

Its rank  is 2  observable, which means if you know the input ut, in this case  a, and your measurement is the position, in this case x1t, You may determine not only x1(t) but also x2(t)

- Case_2: If your measurement is the position, x2t, then  C=01, the observability matrix Q is

Q= CCA= CCA=0100
It’s rank = 1, which is not observable. As U see, in this case, u may not determine the value of x1(t)

- Case _3 : If your measurement is  yt= x1t+ x2t, then  C=11, the observability matrix Q is

Q= CCA= CCA=1101
It’s rank = 2.which is observable. As U see, in this case, U measure  x1t+ x2t, not independently of x1 and x2(t) , the u may  determine the value of x1(t) and x2(t)

- Home assignment: W6.5. In the above case_3)  at a=3ms2, t=2, the measurement

y2=x12+x22=4.

Determine x1t and x2(t)

-----------------------------------%%%%%%%%%%%%%%%%%%%

- Example 1.24 (Inverted pendulum: Observability)

- The measured output is the angle (ϕt. Continue  Ex.1.1 / 1.10 / 1.11/ 1.16 /1.20

xt= 01000-FM000-gL'000gL'10xt+ 01M00ut            1.26

The measured output η(t) is the angle ϕ(t), then 
ηt=-1/L01/L0 xt=1LCx

The Reconstructibility (Observability) matrix Q is

%% Example 1.24 - Observability Check

clear all;clc

syms F M g L s

% L = (J + mL^2)/ mL

A = [ 0 1 0 0; 0 -F/M 0 0; 0 0 0 1; -g/L 0 g/L 0];

C =1/L* [ -1 0 1 0];

Q = [C; C*A; C*A*A; C*A*A*A]

rank(Q)

>> ans

Q =

[   -1/L,                     0,   1/L,     0]

[      0,                  -1/L,     0,   1/L]

[ -g/L^2,               F/(L*M), g/L^2,     0]

[      0, - g/L^2 - F^2/(L*M^2),     0, g/L^2]

And rank(Q) = 3 . Hence it is unobservable system.

- The measured outputs are

The angle ϕt and the position of the carriage st=ξ1(t). Hence the output is a vector as

yt= y1(t)y2t= -1/L01/L01000ξ1ξ2ξ3ξ4

Then the observability matrix has a rank =4, implies the system is observable.

  - Skip

  - Skip

  - Vector Stochastic Process.

One of the major developments in 20 century is the analysis of a Brownian Process, i.e., independent random increments, or independent random walk, which is adapted by Einstein to find “The Relative Theory”. As you know, the famous law is

E=mc2.

To derive  this law, we need the stochastic process, U may remember this topic was dealt in      “Stochstic Control” very little, in the last semester. I will teach this topic once again late of this semester as “Linear Quadratic Gaussian” problem, until then I may skip it.

- Analysis of Linear Control System

  - Introduction

  - The Formulation of Control Problems

    - Introduction

    - The Formulation of Tracking and Regulator Problems

1

- An input variable u(t): ex. motor rotation voltage

- A disturbance variablevp(t) : ex. your finger to touch at the top of a rod

- An observed variables y(t): ex. a sum of position - ξ1+ξ3

- A controlled variable zt: ex.  an angle  of a rod

- A reference variable  r(t): controlled variable z(t): ex. the angle of a rod.

- Tracking problems: z(t) follows a prescribed value. i.e. an angle  may be increased

- Regulation problems: z(t) is remained constant in a long time, i.e., an angle may be zero degree.

- Control design considerations

- The disturbance: It is unpredictable. in general modelled as a random variable

- The plant parameters: is not knows precisely. i.e., the friction

- The initial state may not known

- The observed variable may be contaminated by the noise. In general modelled as a random variable

  - Closed – loop controllers : The basic design objective

  - The stability of control systems

- Design Objective 2.1: The control system should be asymptotically stable

As U see Example 1.4 (Week_5_2), without feedback, the system is unstable. To stabilize the system, U may design a state-feedback so that the closed loop system is stabilized.

- Example 2.6 (Inverted pendulum:  a state feedback continue the  Ex. 1.1 / 1.16 )

From 1-26

xt= 01000-FM000-gL'000gL'10xt+ 01M00ut

Substitute the parameter values (Ex.1.1) to give

xt= 01000-1000-11.6500011.6510xt+ 0100ut

And the state feedback gain by the textbook

BKx=0100 86.8112.21-118.4-33.44ξ1ξ2ξ3ξ4

Then the closed loop e-values are

(-1.9070 +/-3.4229i,   -4.6980 +/- 1.3784i)

Since all e-values have negative real parts, the closed loop system is stable.(Check matlab! There are small difference between the values of text and those of matlab. Which one is correct? I think Matlab is correct!)

- Important issue!!

Since the the equation 1-26 is derived from linearization from (1.16) ~(1.18) at the stationary point,.i.e., ϕt=0, if the point different from the stationary, this state feedback may not be stabilized.

%%%%%%%%%%%%%--------------

The actual system and the linearized system may be different, sometimes totally different. In this example, linearized at  ϕt=0. Since only if  a ~0,

sinϕ= sina+cosa1!ϕ-a+… ~ ϕ

cosϕ= cosa-sin a1!ϕ-a+…~1

The textbook gives the results different ϕ0, the closed loop system may be unstable..

Looks nice!! How the authors got this graph without these days fancy computer? Someday I will plot it. Promise.                                     ---------------------------------%%%%%%%%%%%%%%%%%%%%%%%%