4.2 Observers

- Theorem 4.2

- Theorem 1.32

Here the reconstructibility matrix, which is the same as the observability matrix in this case.

4.3 The Optimal Observer

4.3.1 A stochastic approach to the observer

- Theorem 4.5 The optimal observer.

%%%%%%%%%%----------comments

- The optimal observer gain is determined by (4-105)

- The observer matrix Riccati equation is similar to the controller Riccati equation.

- Linear Quadratic Regulator Riccati equation (controller)

- Controller: -P=ATP+PA+Q-PBR-1BTP,      Ptf=Ptf  (3-130, 131)

- Observer: : Q=AQ +QAT+V1-QCTV2-1CQ,      Q(t0)=Q0  (4-106,107)

We call this relation as “duality property”

- This optimal observer = The minimum mean square linear estimator

= The minimum mean square estimator

= Kalman – Bucy filter

- Example 4.3:

In this example, in the text book the observed variable is constant, But I change the observed variable is not constant but dynamic. Please remember it.

- Modeling

The state  ξt is constant, moving occasionally changes. The measurement η(t) is corrupted.

ξt=aξt+ v1t ,  a=-1;   ξ0=ξ0=1                            (4-123)
ηt=ξt+v2(t)

where v1t, v2(t) are white noise with intensity V1=0.1, V2=0.01 which are independent of each other.

- The dynamic motion without the observer

- The algebraic  Riccati equation,

0= 2AQ+V1 -1V2Q2

Using  the symbolic math and Q≥0, we may get

Q=0.0232

- The optimal observer gain K is

K=QV2= 2.3166

- The observer dynamics   with a different initial condition

ξt=Aξ+Bu+K[y-Cξ]           
=A-KCξ+Bu+Ky
                                                           y=ξ

- Comments:

- The observer ξ  is converge the observed variable ξ after t>~~1.5. Good…

- The initial point of the obverse may be chosen arbitrarily.

- Check the eigenvalue  of the observer system. More importantly as in the cheap control the intensity of the disturbance and noise may be parameterized as

ρ=V1V2

Then you may see the eigenvalue of the observer as varying ρ

- In the lase semester, we learned about in Stochastic control and in Stochastic Identification.

In there

- The least square estimator: in discrete time system

- The Kalman filter: in discrete time system

- Arma . armax   parameter estimation: in discrete time / difference equation.

- In this case, we are dealing with continuous time systems up to now.

- The optimal observer is equivalent to the Kalman estimator.

- More important, this Kalman estimator has better performance over the least square estimator in some sense.

%%%%%%%%%%%%%%---------------------- Comments

In the text book, The author “Kwarkernaak” considered several cases in desigh the optimal observer.

- In order to design the optimal observer, it is necessary to satisfy “Observability matrix should be full rank, i.e., ( rank of the observability matrix = the number  of the states),

- Kwarkernaak is considered this case.

- I will skip these topics

- And I will upload the matlab code… please try to run it and you may change parameters to see what is happening…

- As I said, the estimator / observer is more important nowadays. In artificial intelligence / machine learning and so on … Good Luck!!

-------------------------------------%%%%%%%%%%%%%%%%%%%%%%%%%%%%%